CMPE 240: Advanced Computer Design

Phase 1: MLP Training, Quantization & Weight Export

Complete Code Documentation – Group 1

Zybo Z7-10 | Zynq-7010 SoC | Xilinx Vivado

# 1. Overview

This document explains every section of phase1_mlp.py — the Python script that produces all weight files needed for Phase 2 FPGA deployment. It covers what each section does, why design decisions were made, and what the output files mean.

What Phase 1 Produces

W1.coe / W2.coe / W3.coe → INT8 quantized weights for each fully-connected layer

b1.coe / b2.coe / b3.coe → INT32 pre-scaled biases for each layer

scales.txt → Per-layer FP32 scale factors, read by PS (ARM) for dequantization

mlp_fp32.pth → Full-precision checkpoint for reference / re-quantization

*.npy → NumPy debug arrays (convenient for manual inspection)

# 2. Environment Setup

## 2.1 Requirements

Install the following before running the script:

pip install torch torchvision numpy

Python 3.8+ is required. GPU support (CUDA) is optional but speeds up training. The script auto-detects the device.

## 2.2 Running the Script

python phase1_mlp.py

All output files are written to a newly created phase1_output/ folder in the current directory. Training takes roughly 2–4 minutes on CPU and under 1 minute on a GPU.

# 3. Step 1 – Dataset Preparation

## 3.1 What It Does

The script downloads the MNIST dataset using torchvision.datasets.MNIST. It applies two transforms: dividing pixel values by 255 to get floats in [0, 1], then normalizing using the dataset-wide mean (0.1307) and standard deviation (0.3081). The images are not explicitly flattened here — that happens inside the model's forward() method using x.view(-1, 784).

## 3.2 Why These Values

| Parameter | Value | Reason |
| --- | --- | --- |
| MNIST mean | 0.1307 | Per-pixel mean across the full 60,000 training images |
| MNIST std | 0.3081 | Per-pixel std across the full 60,000 training images |
| Batch size | 128 | Standard mini-batch; balances GPU utilization and gradient noise |
| Test loader | Full 10K at once | Faster evaluation; no shuffling needed for accuracy computation |

# 4. Step 2 – Model Architecture & Training

## 4.1 Architecture

The MLP class defines the network. It has three fully-connected layers:

| Layer | Input | Output | Activation | Parameters |
| --- | --- | --- | --- | --- |
| FC1 | 784 | 64 | ReLU + Dropout(0.2) | 50,240 |
| FC2 | 64 | 32 | ReLU + Dropout(0.2) | 2,080 |
| FC3 | 32 | 10 | None (raw logits) | 330 |
| Total | — | — | — | 52,650 |

## 4.2 Key Design Decisions

Why MLP instead of CNN?

The Zynq-7010 has only 80 DSP slices and 60 BRAM blocks. Even a minimal CNN like LeNet-5 requires far more resources. The MLP with 52,650 parameters fits comfortably after 8-bit quantization (~51 KB), leaving BRAM headroom for control logic.

Why ReLU activation?

ReLU is max(0, x) — just a comparison with zero. In hardware (RTL) this is free: it requires no DSP or LUT-heavy arithmetic. Sigmoid or tanh would require look-up tables or polynomial approximations, consuming significant BRAM and LUTs.

Why Argmax instead of Softmax at output?

Softmax requires exp() and division — both expensive in fixed-point hardware. The hardware only needs the predicted class index (0–9), not probabilities. Argmax is a simple comparator tree over 10 values, which maps to a tiny amount of LUT logic.

Why Dropout during training only?

Dropout randomly zeros activations during training to prevent overfitting. It must be disabled at inference time (model.eval() handles this automatically). Dropout layers are not exported to hardware — the weight files represent the fully-connected layers only.

## 4.3 Training Configuration

| Hyperparameter | Value | Rationale |
| --- | --- | --- |
| Loss function | CrossEntropyLoss | Standard multi-class classification loss; combines LogSoftmax + NLLLoss |
| Optimizer | Adam (lr=1e-3) | Adaptive learning rate; converges faster than SGD on MNIST |
| Epochs | 20 | Sufficient for convergence; early stopping via best-checkpoint saves |
| Dropout rate | 0.2 | Light regularization; MNIST is simple so heavy dropout is counterproductive |
| Target accuracy | ≥ 97% | Project specification requirement |

## 4.4 Best Checkpoint

After every epoch the test accuracy is evaluated. The best-accuracy model state is saved to memory. At the end of training, this best state is restored before quantization. This prevents a slightly worse final epoch from affecting the exported weights.

# 5. Step 3 – INT8 Quantization

## 5.1 What Quantization Does

Quantization reduces each 32-bit floating-point weight to an 8-bit integer. This achieves roughly 4× memory compression (52,650 weights: 210 KB FP32 → 52 KB INT8), allowing all weights to fit in the Zynq-7010's on-chip BRAM. The hardware MAC units also become simpler (8-bit × 8-bit multipliers vs 32-bit float multipliers).

## 5.2 Symmetric PTQ Formula

The script uses symmetric per-layer quantization with zero-point = 0. For each layer:

scale  = max( |W_min|, |W_max| ) / 127

W_q    = clamp( round(W / scale), -128, 127 )

Why symmetric (zero-point = 0)?  The hardware MAC inner loop then becomes a pure INT8 multiply-accumulate. With a non-zero zero-point, every multiply would need an extra subtraction, adding DSP operations and control complexity.

## 5.3 Bias Quantization (INT32)

Biases are stored as INT32, not INT8. The bias is pre-scaled using the same layer scale:

b_q = clamp( round(b / scale), INT32_MIN, INT32_MAX )

In the hardware datapath, the bias is loaded directly into the 32-bit accumulator before the MAC loop begins. This avoids any need for the hardware to know the scale factor during inference — the scale is only needed in the PS (ARM) when interpreting the final logits.

## 5.4 Accuracy Verification

After quantization, the script simulates the quantized model in NumPy by dequantizing the INT8 weights back to float (W_f = W_q × scale) and running inference on the full 10,000-image test set. The simulated accuracy should stay within about 1–2% of the FP32 baseline. If it drops below 95%, the proposal specifies applying Quantization-Aware Training (QAT).

# 6. Step 4 – .coe File Export

## 6.1 Xilinx .coe File Format

Vivado's Block Memory Generator reads .coe files to initialize BRAM contents at synthesis time. The format is:

memory_initialization_radix=16;

memory_initialization_vector=

7F, 3A, FF, 80, 01, ...,

..., 4E;

Each entry is one byte (for INT8 weight files) or four bytes (for INT32 bias files), written in hexadecimal two's-complement. The semicolon terminates the last entry.

## 6.2 Output Files Reference

| File | Contents | Data Type | Size |
| --- | --- | --- | --- |
| W1.coe | FC1 weights (784 × 64, row-major) | INT8 | ~49 KB |
| W2.coe | FC2 weights (64 × 32, row-major) | INT8 | ~2 KB |
| W3.coe | FC3 weights (32 × 10, row-major) | INT8 | 320 B |
| b1.coe | FC1 biases (64 values) | INT32 | 256 B |
| b2.coe | FC2 biases (32 values) | INT32 | 128 B |
| b3.coe | FC3 biases (10 values) | INT32 | 40 B |
| scales.txt | Per-layer scale factors | FP32 | 3 lines |

## 6.3 Row-Major Storage

Weights are stored row-major: for a layer with N outputs and M inputs, row i contains the M weights connecting all M inputs to output neuron i. The FPGA FSM iterates addresses sequentially, so the memory layout matches the access pattern of the MAC loop.

## 6.4 Using .coe Files in Vivado

In the Phase 2 Vivado project, configure each Block Memory Generator IP as follows:

- Set 'Memory Type' to Single Port ROM (or Simple Dual Port RAM if writeable)

- Set 'Width' to 8 bits (for weight BRAMs) or 32 bits (for bias BRAMs)

- Under 'Initialization', enable 'Load Init File' and select the corresponding .coe file

- The BRAM will be pre-loaded at synthesis; no runtime loading is needed

# 7. Hardware Handoff Checklist

Before starting Phase 2, verify all of the following:

| Check | How to Verify | Expected Result |
| --- | --- | --- |
| FP32 accuracy | Script prints final accuracy | ≥ 97.0% |
| INT8 accuracy | Script prints simulated accuracy | ≥ 95.0% |
| W1.coe exists | ls phase1_output/W1.coe | ~49 KB file |
| scales.txt exists | cat phase1_output/scales.txt | 3 non-zero float values |
| .coe format | Head first line of any .coe | memory_initialization_radix=16; |
| Weight range | Inspect .npy with numpy | All values in [-128, 127] |
| Bias range | Inspect .npy with numpy | All values fit INT32 |

# 8. Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| FP32 accuracy < 97% | Model under-fitting | Increase epochs to 30; verify normalization values |
| INT8 accuracy < 95% | Large weight ranges causing quantization error | Apply QAT: retrain with fake-quantized weights |
| Training very slow | Running on CPU without GPU | Install CUDA PyTorch; or reduce to 15 epochs |
| ImportError: torchvision | Package not installed | pip install torchvision |
| .coe file missing | Script crashed early | Check console for Python traceback |
| Vivado rejects .coe | Line-ending mismatch (Windows) | Open file in VS Code and change to LF endings |

# 9. Annotated Script Walk-Through

Below is a section-by-section reference to the script structure. Each section number corresponds to a comment block in phase1_mlp.py.

| Section # | Code Block | Purpose |
| --- | --- | --- |
| 0 | Config constants | Centralized hyperparameters (epochs, LR, batch size, targets) |
| 1 | Dataset loading | Download MNIST, apply transforms, create DataLoaders |
| 2 | MLP class definition | Defines the 784→64→32→10 network with dropout |
| 3 | evaluate() function | Reusable function: returns % accuracy on any DataLoader |
| 4 | Training loop | 20-epoch Adam training with best-checkpoint save |
| 5 | symmetric_quantize() | Converts FP32 array to INT8 with returned scale factor |
| 6 | Accuracy verification | NumPy simulation of INT8 inference on full test set |
| 7 | write_coe_int8/int32() | Generates Xilinx BRAM initialization files |
| 8 | Summary printout | Final accuracy, memory usage, BRAM block estimate |

# 10. Expected Console Output

When the script runs successfully, you should see output similar to the following:

============================================================

STEP 1: Downloading and Preparing MNIST Dataset

============================================================

Training samples : 60,000

Test samples     : 10,000

...

STEP 2: Training FP32 MLP

Epoch  1/20  |  Loss: 0.4312  |  Test Acc: 93.21%

...

Epoch 20/20  |  Loss: 0.0881  |  Test Acc: 97.84% ★ best

✓ Final FP32 accuracy (best checkpoint): 97.84%

✓ Target 97.0% reached!

STEP 3: Symmetric INT8 Post-Training Quantization

fc1: shape=(64, 784)  scale=0.007843  ...

INT8 accuracy : 97.12%

Accuracy drop : 0.72%

STEP 4: Exporting .coe Files

Wrote W1.coe   (50176 values, ~49 KB)

...

PHASE 1 COMPLETE – All targets met. Hand off to Phase 2.

CMPE 240 Group 1  |  Zybo Z7-10 FPGA MLP Accelerator  |  Phase 1 Documentation